All Poly Brush Truck

Specifications

Prepared for:

Stock

Date: 4/21/2014

901 Commerce Road

Luverne, MN  56156

1-800-344-2059

300-Gallon All Poly Brush Truck

Specifications

INDEX to Specifications

Standard:

Section 1:	Apparatus Body & Booster Tank

Section 2:	Booster Tank Piping, Fills, & Gauges

Section 5:	Apparatus Components

Section 6:	Apparatus Body Compartments

Section 8:	Grab Rails & Foot Steps

Section 9:	Electrical Equipment

Section 10:	Emergency Siren & Lighting Equipment

Section 11:	Painting, Lettering, Striping, and Signs

Section 12:	Corrosion Protection & Mud Flaps

Section 13:	Pump & Plumbing

Section 14:	Hose Trays, Cross Lays, & Pre-connects

Section 15:	Pike Poles and Axes

Section 16:	Hose Reel

Section 17:	Chassis Accessories

Section 18:	Loose Equipment

Section 19:	Chassis

STANDARD

Section 1:	Apparatus Body & Booster Tank

1.04

The 300 gallon tank shall be an APR 300 Gallon polypropylene baffled tank constructed of 1/2" thick high-impact, polypropylene sheet stock. The material shall be of a certified, high quality, non-corrosive, stress relieved thermoplastic, black in color, and UV stabilize for maximum protection.

The Tank Shall Carry a Lifetime Warranty from Its Manufacturer

The tank shall have a combination vent and manual fill tower. The fill tower shall be constructed of 1/2" PT2E polypropylene and shall have a standard dimension of 8" x 8" with a molded cover. The tower shall be located on the curb side rear corner of the tank. Inside of the fill tower approximately 2" down from the top shall be fastened a combination vent/overflow pipe which shall be of standard schedule 40 polypropylene pipe with minimum ID of 3" designed to run vertically through the tank. The transverse swash partitions shall be manufactured of 3/8" PT2E polypropylene which shall interlock with a longitude partition constructed of 1/2" polypropylene.

There shall be one (1) 1/4” deep X 6 1/2” diameter recess in floor to act as sump.

There shall be a 1" FNPT female tank drain located on the rear tank wall with a quarter turn valve.

1.09.2

Folding aluminum access ladder, there shall be two (2) folding aluminum access ladders featuring 1-1/4” diameter knurled tube rails and serrated rungs, shall be located at the rear of tank, one (1) on the driver and one (1) on the passenger side for over-head tank filling and access to the hose bed/trays.

Section 2:	Booster Tank Piping, Fills, & Gauges

2.10

One (1) Innovative Controls, SL Series 14 LED light tank level gauge will monitor the water tank level and be located on the pump control panel. The tank level gauge shall indicate the water level on an easy to read LED display and show increments of 1/8 of a tank.

Section 5:	Apparatus Body and Components

5.00

Apparatus construction shall include a dedicated body sub-frame. The body sub-frame shall be integral to the tank cradle and constructed using extruded aluminum tubing of .25” wall thickness. The body sub-frame shall be designed to undergird the body structure and to provide maximum support for the weight of the body structure and all stored equipment. The apparatus body shall be attached to the body sub-frame using rigid fasteners isolated from the body by fitted rubber bushings.  This mounting system will provide positive attachment of the body to the body sub-frame while allowing sufficient range of movement between the body and the body sub-frame. The body design shall be enclosed on all sides and shall incorporate closed wheel wells and finished storage compartments per the specification.

An all aluminum cradle shall be engineered and constructed to interface between the chassis frame and the copolymer tank and body.  The cradle shall be constructed using extruded aluminum tubing of .25” wall thickness and extruded aluminum flats .375” thick.  The bottom of the tank shall be supported at the perimeter by the cradle frame and shall be supported throughout the under surface of the tank floor by a grid structure of cradle cross members.  The cradle cross members shall be spaced to restrict unsupported portions of the tank between cross members to a maximum of 550 square inches. There shall be cushioned rubber extrusions placed over all tank support areas to isolate the tank from the aluminum cradle. The rubber cushion material shall be extruded to a profile matching the cradle structure material to prevent the rubber material from moving from its installed position.  The tank shall be captured front and rear as well as side-to-side by additional cradle structure to prevent the tank from shifting during vehicle operation.  The tank shall be affixed to the cradle utilizing hat channel mounting brackets constructed of .25” thick stainless steel.  The channels shall be mounted from beneath the center fore-to-aft members of the cradle cross members, surrounding these members and shall be bolted directly to the bottom of the tank thereby securing the tank to the cradle.  This mounting system will provide a free-floating connection of the tank to the cradle which allows the chassis frame’s normal movement and twist to introduce no stress upon the tank.

Fenders shall be integral with the side of the body. Fender wells shall be constructed with full circular copolymer polypropylene thermoplastic inner liners for ease of cleaning and maintenance.

5.00.1.9

A welded tubular assembly to be install around the walkway area to protect personnel from branches and other obstacles. Constructed of 2” diameter D.O.M. steel tubing having a yield strength of 40ksi. (2) man wells that are 22”D x 28”W x 6ft 6in high. The man wells will have a vented extruded aluminum floor. A tubular steel door with a painted center panel will swing “in word” and have a positive latch. The tubular structure will be powder coated black for maximum protection. The man well area will have step area lights that will be controlled from a switch in the cab.

5.01

The entire apparatus body shall be fabricated from non-corrosive, stress relieved virgin copolymer polypropylene thermoplastic material. All exterior body joints and seams shall be “extrusion welded”. All welds shall conform to DVS and AWS standards. Extrusion welds must provide a full penetration with one pass. “V” welds are not to be used for any structural assemblies. All joints, seams and welds shall be tested for integrity and shall be certified to be free from defects. The minimum thickness of the body components shall not be less than 3/4" thick. Exterior body panels that are 1/2" thick or less are not permitted.

Fenders shall be integral with the side of the body. Fender wells shall be constructed with full circular copolymer polypropylene thermoplastic inner liners for ease of cleaning and maintenance.

5.02

There shall be black rubber molded fender flares installed on the wheel wells to prevent splash and to give the body a pleasing appearance. The fasteners shall not be exposed to the exterior of the fender flares.

Section 6:	Apparatus Body Compartments

6.00.5.9

Compartment, one (1) full height "sweep out" style compartment shall be provided on the driver's side ahead of the rear wheels. The compartment shall be integral to the apparatus body and shall be constructed using white copolymer material.  The compartment shall have approximated dimensions of 25 wide by 56” tall by 18 to 23 deep. The compartment shall have a 22” R-O-M anodized aluminum roll-up door, door activated LED compartment lights, corrosion resistant vents, black Turtle Tile plastic dry decking, and floor drains.

6.01.5

Compartment, one (1) full height "sweep out" style compartment shall be provided on the passenger's side ahead of the rear wheels. The compartment shall be integral to the apparatus body and shall be constructed using white copolymer material.  The compartment shall have approximated dimensions of 25” wide by 56” tall by 18 to 23” deep. The compartment shall have a 22” R-O-M anodized aluminum roll-up door, door activated LED compartment lights, corrosion resistant vents, black Turtle Tile plastic dry decking, and floor drains.

6.03.5

Compartment, one (1) horizontal "sweep out" style compartment shall be provided on the driver's side over the rear wheels. The compartment shall be integral to the apparatus body and shall be constructed using white copolymer material.  The compartment shall have approximated dimensions of 73” wide by 30” tall by 23" deep. The compartment shall have a 70” R-O-M anodized aluminum roll-up door, door activated LED compartment lights, corrosion resistant vents, black Turtle Tile plastic dry decking, and floor drains.

6.04.5

Compartment, one (1) horizontal "sweep out" style compartment shall be provided on the passenger's side over the rear wheels. The compartment shall be integral to the apparatus body and shall be constructed using white copolymer material.  The compartment shall have approximated inside dimensions of 73” wide by 30” tall by 23" deep. The compartment shall have a 70” R-O-M anodized aluminum roll-up door, door activated LED compartment lights, corrosion resistant vents, black Turtle Tile plastic dry decking, and floor drains.

6.05.5

Compartment, one (1) horizontal "sweep out" style compartment shall be provided on the center rear of body. The compartment shall be integral to the apparatus body and shall be constructed using white copolymer material.  The compartment shall have approximated inside dimensions of 48” wide by 28” tall by 32" deep. The compartment shall have a 48” R-O-M anodized aluminum roll-up door, door activated LED compartment lights, corrosion resistant vents, black Turtle Tile plastic dry decking, and floor drains. This compartment is open to the rear side compartments with a 19 x 21 opening, suitable for long items such as back boards etc

6.05.9

Behind the wheel storage, there shall be one compartment on both the driver and passenger side behind the rear wheels. The compartment shall be 22 wide x 17 tall x 18 deep with an aluminum slam door, D-ring latch, and LED lighting.

6.10

Shelving, four (4) adjustable shelves made of 3/16" smooth aluminum shall be installed with Unistrut adjustable brackets, two (2) in the driver's side front compartment and two (2) in the passenger’s side front compartment ahead of the rear wheels. The vertical location of the shelves shall be per the department’s instructions.

6.10.1

Standard slide-out shelf, one (1) horizontal slide-out shelf, approximately 45” wide shall be installed in the rear compartment on the back of the apparatus. The slide-out shelf shall be constructed with roller bearing equipped, linearly sliding mechanisms which are lockable in both the “In” and “Out” positions. The shelf shall be equipped with reflective striping to be visible when in the extended position.

Section 8:	Grab Rails & Foot Steps

8.00.9

Tubing/grab rail, there shall be one (1) 1 3/4” grab rail supplied on the top of the apparatus on both the driver and passenger side. The rail shall be powder coated black.

8.09.2

A 2” diameter nerf bar shall be installed under the apparatus to protect the body against debris and to give it a pleasing appearance.

Section 9:	Electrical Equipment

9.00.1

The body and chassis shall be wired as independent modules and connected as a completed unit at the final assembly via electrical connectors located in the electrical compartment. Seals shall be provided on each individual wire and the assembly as a whole. All wiring for the apparatus body shall be within a temperature resistance harness. All wires in each harness shall be color- coded. Wiring shall be run along structural rails and tied in a neat and orderly manner.

9.00.2

The electrical junction box for all apparatus connections, relays, circuit breakers, etc., will be located in the face of the forward passenger side compartment. It shall be of .125” aluminum diamond pattern polished treat plate construction with an aluminum hinge. The apparatus circuits requiring load protection shall utilize sealed relays and automatic reset circuit breakers.

9.01.2

LED Clearance lights, there shall be a total of nine (9), seven (7) red LED lights installed at the rear of the apparatus, and two (2) amber LED lights installed on the front left and right of the apparatus.

9.02

Reflectors shall be installed per Department Of Transportation specifications.

9.03.4.9

Two (2) Whelen model M6 Series, red LED stop/tail/turn lights shall be supplied and installed on the rear of the apparatus. Individual lights shall measure 6 1/2" wide by 4 1/8" tall. Light assemblies shall be provided in chrome plated housing. Each light assembly shall be installed using a gasket between the housing and body.

9.06

Compartment "door ajar" warning light, there shall be a "red” pilot light that is visible to the driver and it shall illuminate when a compartment door is open.

9.08

Illuminated license plate bracket, there shall be one (1) illuminated license plate bracket installed at rear of the apparatus.

Section 10:	Emergency Siren & Lighting Equipment

10.00

Control Center, all accessory and emergency lighting shall be controlled at a master control center in the cab. The control center shall contain lighted rocker switches and shall be heavy duty rated to service the maximum imposed electrical load. The control center shall have lighted identification plates on a non-glare panel face that clearly identify each switch and its function. The control center shall control all warning lights, scene lights, and include the "Master On" and "Open Door" indicator lights.

10.01.3.1.1.9

Light bar, the upper level warning package shall be fully NFPA compliant and shall be certified by the lighting component manufacture to meet all requirements. The front and front side zones shall be covered by one (1) Whelen Freedom Series, Super-LED low-profile 55” light bar. Activation of the light bar shall be via the control center located in the cab.

10.02.1.1

Siren, one (1) Whelen model 295SLSA1, 200 watt, six function Class A electronic siren shall be mounted in the chassis cab in a location convenient to the driver. The electronic siren shall include full function, 17 Scan-Lock siren tones, and hard wired microphone. The siren control head shall be lighted for easy night operation.

10.03.1.1

Speaker, one (1) Cast Products 100-watt polished aluminum siren speaker shall be “flush” mounted through the chassis front bumper.

10.04.4.1.9

LED Flashing lights, the lower level warning package shall be fully NFPA compliant. Two (2) Whelen model M6V2R combination warning/area lights, flush mounted red LED flashers with black bezels shall cover each zone. These lights shall be 6” wide by 4” tall and be mounted at the front of the chassis. The front zone shall be covered by two (2) lights and rear zone shall be covered by two (2) lights.

10.05.5.1.9

Combination warning/scene light, there shall be six (6) Whelen M6V2R combination warning/scene lights installed, two (2) on the driver, two (2) on the passenger, and two (2) on the rear of the apparatus. These lights will cover both the upper warning lights and scene lights. Controls for the lights shall be located in the chassis cab.

10.06.4.2.9

LED Intersection lights, the lower level warning package shall be fully NFPA compliant and shall be certified by the lighting component manufacture to meet all requirements. There shall be two (2) Whelen model M6 warning lights, surface mounted red LED flashers with chrome bezels that cover each of the side zones. These lights shall have approximate dimensions of 6" wide by 4" tall. One of the lights shall be mounted at the rear of the body, and one shall be mounted on the front corner of the chassis for use as an intersection style light.

10.07.9.2

Deck lights, there shall be four (4) PTL5040 LED deck lights installed. There shall be one (1) on each side of headache rack mounted at the highest point and facing outward and there shall be two (2) facing the rear mounted on the outside corners of the rear guard rails. The forward lights are controlled by a switch in the cab and the rear lights are controlled by a switch on the rear of the apparatus.

10.08

Backup alarm, there shall be a backup alarm installed. Alarm shall be energized automatically when the chassis is placed into reverse.

Section 11:	Painting, Lettering, Striping, and Signs

11.01

The entire apparatus body will be sanded, cleaned and etched with PPG epoxy primer and primed with PPG urethane primer filler. The apparatus body will be hand sanded and be painted one (1) solid color of red using a PPG Deltron base coat/clear coat paint to match the chassis paint color. A two (2) ounce container of matching touch-up paint, with applicator brush, shall be supplied for each color of the finished apparatus body paint upon completion of the apparatus.

10-YEAR WARRANTY ON PAINTED BODY PARTS

11.05

Gold leaf vinyl lettering with a black back shadow shall be provided on the chassis doors per customer instructions. Clear protective vinyl shall be installed on all gold leaf vinyl lettering.

11.05.2

Gold leaf vinyl lettering with a black back shadow shall be provided for the customer unit number on the left and right chassis fenders.  Clear protective vinyl shall be installed on all gold leaf vinyl lettering.

11.06.1.2

White reflective striping, the body and chassis shall have a 4" wide and a 1" wide white reflective stripe on each side and front of the apparatus per NFPA 1901 standards.

11.06.2.1

Retro-reflective material, per NFPA 1901 standards any door of the apparatus designed to allow persons to enter or exit the apparatus shall have a minimum of 96 square inches of retro-reflective material affixed to the inside of the door.

11.07.5.2

Chevron, 100 percent of the rear of the apparatus shall include red and fluorescent yellow Chevron retro-reflective striping a minimum of 6” wide.

11.20

There shall be a permanent plate located in the chassis cab with the following information:

- Quantity and type of fluids used in the vehicle. This plate shall include engine  oil, engine coolant, chassis transmission fluid, pump transmission fluid, drive axle lubrication fluid, air conditioning refrigerant, air conditioning lubrication oil, and power steering fluid.

- Front and rear cold tire pressures.

- Number of personnel the vehicle is designed to carry shall be located in an area visible to the driver.

- Height and length of the vehicle in feet and inches

- Gross vehicle weight rating (GVWR) in pounds

Section 12:	Corrosion Protection & Mud Flaps

12.01

All fasteners used in securing components on or to the body shall be stainless steel.

12.02

Mud flaps, there shall be two (2) mud flaps installed on rear of apparatus behind the rear wheels.

Section 13:	Pump & Plumbing

The pump shall be installed behind the walkway area. All controls will accessible form this location.

13.09.1

The pump shall be a Hale, model HP300X, engine driven pump. Pump features include a high-strength aluminum alloy casing, silicone bronze impeller and wear rings, stainless steel impeller shaft, and direct drive.

Pump Ratings: 400 GPM @ 20 PSI

75 GPM @ 120 PSI

The engine shall be a four cycle air cooled 18 HP, V-Twin, Briggs and Stratton, model Vanguard gasoline engine, with integral fuel pump, 12- volt electric start, and manual start. The fuel will be connected to the chassis fuel system.

13.09.2.1

The pump control panel shall include a throttle lever, priming lever, Master On/Off switch, start push button, and choke control, low oil pressure warning light, and pressure gauge. The pump primer shall be a combination vacuum type exhaust primer and a spark-arresting muffler.

13.09.4.1

Tank to pump line, there shall be a 2 1/2" tank to pump line provided from the water tank to the pump. The line shall have a   2 1/2" Unibody swing out valve with galvanized piping and victaulic coupling. The flex connections installed between the pump and water tank shall give the plumbing system flex, thus minimizing stress on the line. A handle provided in the pump compartment shall control the valve.

13.09.5.1

Tank fill/pump re-circulating line, there shall be a 1.5" tank fill/pump re-circulating line provided from the pump to the water tank. The line shall have a 2" Unibody swing out valve with 1.5" plumbing. The flex connections installed between the pump and water tank shall give the plumbing system flex, thus minimizing stress on the line. A handle provided in the pump compartment shall control the valve.

13.09.6.1

Pump drain, there shall be one (1) pump drain provided.

13.09.7.1

Suction, there shall be one (1) 2.5" suction pipe provided at the pump for drafting or direct fill. The suction shall be equipped with a 2.5” NPT male X 2 1/2" NST female chrome plated swivel adapter with screen, and a 2 1/2" chrome plated plug and chain.

13.70.1

Man well pre-connect, there shall be two (1) 1” NST male, gated pre-connects, one (1) on the driver side and one (1) on the passenger side with a 1” swing out valve and 1” plumbing.

Section 15:	Hard Suction & Ladder Carriers

15.51

Storage compartment, there shall be a full length compartment provided above the street side compartments.

15.52

Storage compartment, there shall be a full length compartment provided above the curb side compartments.

15.53

Hose bed, there shall be a full length hose bed supplied above the tank and rear compartment.

Section 16:	Hose Reel

16.05

There shall be one (1) electric rewind hose reel with 100' of 1" booster lite hose and stainless steel double captive hose rollers installed in the rear compartment. Hose reel shall be gated with a 1” Unibody swing out valve and controlled at the pump panel. The hose reel rewind button shall be located at the rear of the apparatus.

Section 17:	Chassis Accessories

17.00.9

Wheel inserts, there shall be a stainless steel insert installed in the center of the wheels to give it a pleasing appearance.

17.31

There shall be a 2” receiver hitch tube, bumper, and dual tow ring assembly installed on the rear of the apparatus.

17.32.9

There shall be one (1) Weis Equipment Heavy Duty front bumper / grille guard installed on the front of the chassis and shall include a 2” receiver tube. The bumper/grille guard shall be powder coated black and fitted with a 2” NPT flange for future bumper monitor

17.33

Super singles, the chassis shall be equipped with super single tires and rims. Tires shall be 36” x 13.5” x 19.5”, 16 H-ply rating of 6400lbs per wheel and shall be mounted on a 19.5” x 9.75” Steel black powder coated wheels. The tires and rims will carry the factory GVW of the chassis.

17.34

Chassis lift, there shall be a 4” suspension lift installed on the chassis to accommodate the super single tires and wheels.

17.36

Tire rack, there shall be one (1) tire rack supplied on the top of the tank for the spare tire.

17.40

Extended fender flares, there shall be extended fender flares added to the chassis factory fender flares to accommodate the debris from the oversized super singles.

17.52

Chassis steps, there shall be one (1) set of Luverne Truck Equipment Baja steps installed on the chassis for ease of entry. The Baja Step features a 3” stainless steel accent tube. The step plates are constructed from 10 gauge stainless steel. The entire no-rust assembly is powder coated black.

Section 18:	Loose Equipment

18.03.2

There shall be two (2) 1” x 6’ whip lines supplied by the manufacturer. Whip lines shall have NST connections.

18.04.6

Nozzle, there shall be one (1) Task Force Tips model # DS1040P ball shut off nozzle provided. The selectable, dual gallonage nozzle shall be furnished with flow settings of 10 and 40 GPM at 100 PSI and produce fog and straight stream patterns. The nozzle body shall be constructed from hardcoat anodized aluminum alloy, utilize a stainless steel ball shut off valve with a quick change polymer valve seat. An integral pistol grip handle shall be positioned directly below the valve handle. This nozzle shall have a “twist off” position for positive shut off. The nozzle shall be furnished with a 1" female NH swivel rocker lug inlet and designed to accept the Task Force Tips FJ-MX-D FoamJet foam aspirating attachment. The unit shall have a unique serial number and be covered by a written five-year warranty.

Section 19:	Chassis

19.01

Midwest Fire Equipment furnished 2015 Ford F-550 per specification attached: